AXI4 lite bugs

1. Compile with vcs

->Correct the rtl path in and the extension of the design file in makefile

2.

-> Added full path in Makefile

3.

-> Import  uvm_pkg::* in interface file

4.

-> Added full path in tb_pkg.sv for all components

6. Changed output_monitor to axi_monitor in env and pkg

7. In scoreboard commented analysis_fifo for output monitor

8.  clk and rst_n not instantiated with the dut

->  In tb_top.sv declare clk  and in axi_if.sv declare rst_n signal

Compilation error free

Run_reset test

The sequence was calling get_response() to wait for a response transaction from the driver.

But, the driver was not sending any response using put_response().

Because of this, the sequence remained blocked, continuously waiting for a response.

This blocking condition eventually resulted in a simulation timeout error.

The issue was resolved by modifying the driver to send the response transaction back to the sequence using put_response().

Waveform

1. Reset_test

2. Baisc_write_test

3. Basic_read_test

4. Protocol_test

5. Wstrb_test

6. Irq_test

7. Register test

8. Concurrent test

9. Stress test

In the address and data states (for both read and write operations), the next state transition was controlled only by the ready signal.

However, according to the protocol specification, the state transition should depend on both valid and ready signals.

To correct this, the code was modified to include the valid signal along with the ready signal for controlling the state transitions.