AXI4-lite_LED_SevenSeg_IRQ

Overview

- Memory-mapped AXI4-Lite slave peripheral

- Provides control and status interface for external hardware

- Designed for simple register-based communication

Core Functionalities

- Controls 8-bit LED output

- Controls 8-bit seven-segment display

- Generates edge-triggered interrupt (IRQ)

- Provides software-accessible registers for control and status

AXI Interface Features

- Independent read and write channels

- Standard VALID/READY handshake mechanism

- One outstanding read transaction

- One outstanding write transaction

Register Access Features

- Memory-mapped register interface

- Byte-wise write operations using WSTRB

- Synchronous register updates

- Address decoding based on word-aligned addressing

Supported Features

- AXI4-Lite compliant interface

- Independent read/write handling

- Single outstanding transaction per channel

- Byte-enable support (WSTRB)

- Edge-triggered interrupt generation

- W1C interrupt clearing

- Fully synchronous design

Unsupported / Limitations

- No burst transfers

- No multiple outstanding transactions

- No out-of-order responses

- No unaligned accesses

- No support for AXI IDs

Bugs/ Issues identified

During Compilation

1. Compile with vcs

->Correct the rtl path in and the extension of the design file in makefile

2.

-> Added full path in Makefile

3.

-> Import  uvm_pkg::* in interface file

4.

-> Added full path in tb_pkg.sv for all components

5. Changed output_monitor to axi_monitor in env and pkg

6. In scoreboard commented analysis_fifo for output monitor

7.  clk and rst_n not instantiated with the dut

->  In tb_top.sv declare clk  and in axi_if.sv declare rst_n signal

-> Compilation error free

During simulation

Run_reset test

- The sequence was calling get_response() to wait for a response transaction from the driver.

- But, the driver was not sending any response using put_response().

- Because of this, the sequence remained blocked, continuously waiting for a response.

- This blocking condition eventually resulted in a simulation timeout error.

- The issue was resolved by modifying the driver to send the response transaction back to the sequence using put_response().

Waveform

1. Reset_test

2. Baisc_write_test

3. Basic_read_test

4. Protocol_test

5. Wstrb_test

6. Irq_test

7. Register test

8. Concurrent test

9. Stress test

- In the address and data states (for both read and write operations), the next state transition was controlled only by the ready signal.

- However, according to the protocol specification, the state transition should depend on both valid and ready signals.

- To correct this, the code was modified to include the valid signal along with the ready signal for controlling the state transitions.

10. Unaligned addr test

- Since for write and read logics in design are considering only [5:2] bits, even though unaligned  addresses are targeted, for few addresses registers are getting access

- For non-specified register addresses. Ex: for 0x44  address sevenseg register is getting accessed and for 0x40 address led register is getting accessed.

Coverage report